UMass Amherst

Adaptive Use Bridge Project

Southern Vermont (1906) Bridge Load Test 1 Results

Bridge diagram and node labeling. Dial gages located at points 1, 2 and 3.

Test #1. Truck's front axle placed at location 3.

| Gage # | Zero (in) | Disp 3 (in) | Net 3 (in) |
| --- | --- | --- | --- |
| 1 | 0.0035 | 0.0335 | 0.0300 |
| 2 | 0.0035 | 0.0400 | 0.0365 |
| 3 | 0.0000 | 0.0400 | 0.0400 |

Test #2. Truck's front axle placed at location 3.

| Gage # | Zero (in) | Disp 3 (in) | Net 3 (in) |
| --- | --- | --- | --- |
| 1 | 0.0035 | 0.0310 | 0.0275 |
| 2 | -0.0025 | 0.0380 | 0.0405 |
| 3 | -0.0015 | 0.0355 | 0.0370 |

Test #3. Moving Load; Front axle placed at location 1, 3 and 4.

| Gage # | Zero (in) | Disp 1 (in) | Net 1 (in) | Disp 3 (in) | Net 3 (in) | Disp 4 (in) | Net 4 (in) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.0040 | 0.0250 | 0.0210 | 0.0310 | 0.0270 | 0.0090 | 0.0050 |
| 2 | -0.0035 | 0.0235 | 0.0270 | 0.0365 | 0.0400 | 0.0150 | 0.0185 |
| 3 | -0.0020 | 0.0185 | 0.0205 | 0.0355 | 0.0375 | 0.0180 | 0.0200 |

Test #4. Moving Load; Front axle placed at location 1, 3 and 4.

| Gage # | Zero (in) | Disp 1 (in) | Net 1 (in) | Disp 3 (in) | Net 3 (in) | Disp 4 (in) | Net 4 (in) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.0040 | 0.0240 | 0.0200 | 0.0295 | 0.0255 | 0.0075 | 0.0035 |
| 2 | -0.0015 | 0.0250 | 0.0265 | 0.0380 | 0.0395 | 0.0155 | 0.0170 |
| 3 | -0.0010 | 0.0185 | 0.0195 | 0.0350 | 0.0360 | 0.0170 | 0.0180 |

Summary of results. All displacements are inches.

|  | Front axle at 1 |  |  | Front axle at 3 |  |  | Front axle at 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test | gage 1 | gage 2 | gage 3 | gage 1 | gage 2 | gage 3 | gage 1 | gage 2 | gage 3 |
| 1 | - | - | - | 0.0300 | 0.0365 | 0.0400 | - | - | - |
| 2 | - | - | - | 0.0275 | 0.0405 | 0.0370 | - | - | - |
| 3 | 0.0210 | 0.0270 | 0.0205 | 0.0270 | 0.0400 | 0.0375 | 0.0050 | 0.0185 | 0.0200 |
| 4 | 0.0200 | 0.0265 | 0.0195 | 0.0255 | 0.0395 | 0.0360 | 0.0035 | 0.0170 | 0.0180 |
| Mean | 0.0205 | 0.0268 | 0.0200 | 0.0275 | 0.0391 | 0.0376 | 0.0043 | 0.0178 | 0.0190 |
| Max | 0.0210 | 0.0270 | 0.0205 | 0.0300 | 0.0405 | 0.0400 | 0.0050 | 0.0185 | 0.0200 |
| Min | 0.0200 | 0.0265 | 0.0195 | 0.0255 | 0.0365 | 0.0360 | 0.0035 | 0.0178 | 0.0180 |