Windriver Software Setup Document

This document details the steps to be performed for building the Packet generator driver code using the Windriver software.

- Install the board on the PCIe slot (x8 or x16).

- Download the PCIe design and restart the machine.

- Double click on the Driver Wizard icon and click on the Evaluate option.

- Click on the New host driver project option.

- Select the Altera device from the list and click on Next option.

- Click on the option highlighted in the figure below.

- Select the required platform and click OK (I have selected MS Developer Studio 2010 for X86).

- Click OK for the next option

- Save the project in the required folder with the file name pcie_sw.

- Click OK and close the driver wizard.

- Open the project folder and copy the following four files to the root directory (declare.h , pcie_sw_diag_packgen.cpp , reg_defines_packet_generator.h, altpcie_demo.cpp ).

- Copy these remaining files to root directory/x86/msdev_2010 folder (test.pcap, output.txt).

- Double click on the pcie_sw_diag.sln to open the Visual studio 2010 project. The default file structure of Visual studio is shown below.

- Right click and remove the pcie_sw_diag.c file from the file structure as shown below (This file has the default main program).

- Add the four files from the root directory as shown below (Right click pcie_sw_dag -> Add -> Existing Item).

- Select each of the four files from the root directory.

- The file structure after adding the four files is shown below.

- The following files (altpcie_demo.cpp and pcie_sw_diag_packgen.cpp) needs to be complied as C++ code. It can be done by

Right click altpcie_demo.cpp/pcie_sw_diag_packgen.cpp -> Properties -> C/C++ -> Advanced -> Compile as C++ code.

- Now follow the remaining instructions in the user guide (setting up the pcap library) to get the PCIe packet generator working.