Documentation for MODOFC:

A Program for Solving

Optimal Flow Control Problems

Based on MODFLOW Simulation

Version 2.3

May 1, 2003

by

David P. Ahlfeld, R . Guy Riefler

Department of Civil and Environmental Engineering

University of Massachusetts

Amherst, MA 01003

Abstract

MODOFC (MODflow Optimal Flow Control) is a FORTRAN computer program which determines optimal pumping solutions for groundwater flow control problems.  MODOFC couples the USGS MODFLOW simulation program with optimization algorithms. The code can accommodate linear pumping costs, well installation costs, bounds on head and head difference, bounds on streamflow depletion,  bounds on individual and net well pumping rates and bounds on total number of wells. MODFLOW features that can be accommodated include three-dimensional heterogeneous aquifers, confined or unconfined units,  wells screened in single or multiple layers and single or multiple stress periods. MODOFC is designed to utilize existing MODFLOW-96 input files along with a user-created file describing the hydraulic control problem.  MODOFC converts the groundwater flow control problem into an optimization problem by the response matrix method.  MODOFC contains a full implementation of the simplex algorithm. The simplex and branch and bound algorithms are used for mixed binary problems. Sequential linear programming  is used for nonlinear problems.

Table of Contents

1	Introduction	1

2	Summary of Capabilities	2

3	Theory and Methodology	3

3.1	Groundwater Flow Simulation	4

3.2	Linear, Mixed-Binary Optimal Flow Control Problems	4

3.2.1	Constraints on Hydraulic Head	5

3.2.2	Constraints on Pumping	5

3.2.3	Binary Variables	7

3.2.4	Objective Functions	8

3.2.5	Complete Statement of Available Formulation	9

3.2.6	Solving Linear Optimal Flow Control Problems	11

3.3	Nonlinear Optimal Flow Control Problem	13

3.3.1	Solution Algorithm for Nonlinear Problems	13

3.3.2	Avoiding Dewatering During Unconfined Iterations	16

4	Using MODOFC	18

4.1	MODOFC Source Code	18

4.1.1	Dimensioning	19

4.1.2	Compilation of MODOFC	19

4.2	MODOFC Input Features	19

4.2.1	Capture Zones	19

4.2.2	Multiple Well Screens	19

4.2.3	Repeated Optimization	20

4.3	MODFLOW Input Files	20

4.4	Input File	22

4.4.1	Main Section	22

4.4.2	Candidate Wells Section	27

4.4.3	Recharge Balance Constraints Section	31

4.4.4	Total Pumping Constraints Section	31

4.4.5	Head Bound Constraints Section	33

4.4.6	Head Difference Constraint Section	34

4.4.7	Capture Zone Constraints Section	35

4.4.8	Streamflow Depletion Constraints Section	37

4.5	Output Files	38

4.6	Execution	40

5	Sample Problems	40

5.1	Sample Problem I:  CONTAIN	40

5.2	Sample Problem II:  DEWATER	42

5.3	Sample Problem III:  SUPPLY	43

6	References	45

APPENDIX A:  SAMPLE PROBLEMS FOR MODOFC VERSION 2.11

APPENDIX B:  DOCUMENTATION FOR LPSUB VERSION 2.2

APPENDIX C:  LISTING OF SOURCE CODE FOR MODOFC VERSION 2.11

Documentation for MODOFC:

A Program for Solving Optimal Flow Control Problems

Based on MODFLOW Simulation

Version 2.3

May 1, 2003

Copyright 2003 by David P. Ahlfeld

The computer program described in this documentation contains two separate computer code packages.  The first is referred to as MODOFC and includes the computer code modules named ‘modofc23.for’, ‘ofcio23.for’, ‘modflw96.for’, ‘bas5.for’ and ‘modsub.for.’  The second computer code package is referred to as LPSUB and is entirely contained in the computer code module ‘lpsub.for’.

Disclaimer of Warranty

No responsibility is assumed by the code authors, copyright holder or by The  University of Massachusetts for any errors, mistakes, or misrepresentations that may occur from the use of the computer code packages MODOFC and LPSUB. The user of these codes accepts and uses the codes as they are, at the user's own risk, without reliance upon any representation or description concerning the computer codes.  No warranties of any kind, whether statutory, written, oral, expressed, or implied (including warranties of fitness and merchantability) shall apply.

Terms of Code Use and Distribution

- The copyright holder and code manager of the MODOFC and LPSUB computer codes is David P. Ahlfeld of the University of Massachusetts.

- Any person who receives or uses these computer codes should register with the code manager by email to ahlfeld@ecs.umass.edu.

- These codes or any portion thereof may not be sold or otherwise distributed for profit without the express written permission of the code manager.

- The user may make minor modification to the MODOFC code necessary for use in specific applications. Such modifications constitute creation of a new version of MODOFC to which all restrictions described herein apply.

- The user should not make modifications to the LPSUB computer code.

- The user agrees to appropriately acknowledge the authors of this documentation in publications that result from the use of this software or in products that include use of this software.

- Any payments made to obtain these codes represent charges for duplication, handling and registration.

- By acceptance of delivery of these computer codes the user acknowledges and understands the terms on the use and distribution of these codes.

# Introduction

MODOFC solves design problems for the control of groundwater flow.  The design elements considered by MODOFC are the locations and pumping rates at extraction and injection points in a groundwater system.  The program uses optimization techniques to find the best combination of pumping rates and well locations to achieve criteria specified by the user.

The response of the groundwater system to control is modeled using the groundwater flow simulator MODFLOW (McDonald and Harbaugh, 1988 and Harbaugh and McDonald, 1996).  Design criteria can be specified that encompass construction and operation costs, pumping rates and volumes, water level elevations, groundwater flow directions and streamflows.

MODOFC directly couples MODFLOW simulation with an internal optimization solver. MODOFC is designed to utilize standard MODFLOW input files.  Nearly all features present in a MODFLOW simulation can be incorporated into a MODOFC formulation, including confined or unconfined units, wells screened in single or multiple layers and single or multiple stress periods.  A single input file, provided by the user, describes the design criteria to be imposed on the groundwater system.  Upon successful execution, MODOFC reports the optimal placement and pumping rates along with other interpretive information that can be used to better understand the problem.

Solving problems using MODOFC involves two steps.  First, the MODFLOW simulator is constructed to provide a representation of the response of the field system to alternate pumping strategies.  Second, optimization is used to solve for the set of pump rates and well locations which optimize an objective which depends on well installation and pumping rates while satisfying design criteria imposed on the system by the user. This manual describes the steps required to accomplish the second step.  Description of MODFLOW input and model construction is outside the scope of this manual.

Section 2 of this manual provides a concise listing of the capabilities of MODOFC.  Section 3 presents a detailed description of the types of design criteria, in the form of objective and constraint functions, that are available in MODOFC.  This section also contains details on the solution methods used for MODOFC.  Section 4 describes the construction of the MODOFC input files, procedures for execution of the code and a description of output.  Finally, Section 5 presents a series of example problems.

More information on the theory of groundwater management and its application to field problems can be found in Ahlfeld and Mulligan (2000), Ahlfeld, et al. (1995) and Wagner (1995).  Updates of MODOFC are available at www.ecs.umass.edu/modofc.  The development of MODOFC has occurred over several years with the help and suggestions of many individuals.  Charles Sawyer wrote some of the original versions of code for linkage to MODFLOW.  Ann Mulligan extended the range analysis produced by the simplex algorithm. Useful suggestions for enhancements have been contributed by Roger Page, David Dougherty, and George Pinder, and Calvin Chien.

# Summary of Capabilities

This section provides a concise description of the capabilities and features of MODOFC.

Available Objective Function Components

- linear pumping costs

- well installation costs

Available Constraints

- upper and lower bounds on head

- upper and lower bounds on individual well pumping rates

- upper and lower bounds on net pumping in each stress period

- bounds on head difference (or gradient)

- bounds on streamflow depletion modeled with the STR package

- total injection pumping is a fraction of total extraction pumping

- bounds on total number of wells to be used

Available Interfaces with MODFLOW Capabilities

- incorporates MODFLOW-96 version 3.0

- confined or unconfined units

- wells screened in single or multiple layers

- single or multiple stress periods

- wells active at a single rate in any combination of stress periods

- wells active at different rates in any combination of stress periods

- head bounds, head difference bounds, or capture zones active at a single constraint value in any combination of stress periods

- individual pumping rate bounds active in each stress period

- total pumping bounds active in each stress period

- specified injection fraction active in all stress periods

Solution Algorithms Used

- simplex algorithm for linear problems

- sequential linear programming for nonlinear problems

- branch and bound for mixed binary problems

Output Reported

- optimal pumping rates and locations

- binding constraints and shadow prices

- range analysis

- input and run-time error messages

Accuracy and efficiency features

- user selection of perturbation level and capability for variable perturbation level

- dynamic dimensioning coupled with MODFLOW memory needs

- response matrix retained for repeated solution

- capture zone input as a series of line segments

# Theory and Methodology

To define the design problem, the user specifies:  1) a set of locations and time periods to be candidates for well placement and operation;  2) pumping costs and, optionally, construction costs for each candidate well; and 3) requirements on the hydraulic behavior of the system during remediation from among a set of possible constraints on head and pumping rates.  The primary decision variable in the optimization problem solved by MODOFC is the pumping rate at each candidate well in each stress period.  The optimization solver selects those candidate wells that are most effective in satisfying the specified requirements on the system. Those candidate wells that are not selected have their pump rate set to zero by the optimization solver.

This section describes in detail the objective functions and constraints that are available to the user when formulating an optimization problem.  Also presented are the methods used to solve the resulting formulation.

## Groundwater Flow Simulation

MODOFC relies on the program MODFLOW to describe the groundwater system.  MODFLOW determines head with the governing equation;

¶¶xKx¶h¶x+¶¶yKy¶h¶y+¶¶zKz¶h¶z+W=S¶h¶t	(1)

where h is the hydraulic head, Kx is the hydraulic conductivity in the x-direction, Ky is the hydraulic conductivity in the y-direction, Kz is the hydraulic conductivity in the z-direction, W is the flux into or out of the system due to sources or sinks, and S is the storativity.  One possible term included in W is extraction or injection at wells.  For the purposes of MODOFC, (1) describes the relationship between head at any location in the model and the combined pumping at wells, or;

hi,t=hi,t(q)	(2)

where q represents the set of all extraction and injection rates at well locations and operative stress periods and hi,tis the simulated head that results from the stresses described by q. The index i represents a particular location in the model domain while the index t indicates a point in time at the end of stress period t. For steady state problems, the index t can be ignored.  In this case, the head is understood to be that achieved at steady state.  By developing and calibrating a groundwater model with MODFLOW, the user has defined the relationship (2) which is utilized by MODOFC.

## Linear, Mixed-Binary Optimal Flow Control Problems

Design criteria can be incorporated into the optimization procedure by selection of constraints on head, streamflow, pumping, and well installation.

### Constraints on Hydraulic Head

First, an absolute bound may be placed on heads at a location and during a stress period;

hi,t³hi,tl	(3)

or;

hi,t≤hi,tu	(4)

where hi,tland hi,tuare specified lower and upper values at location i at the end of stress period t.  Depending on the direction of the inequality, this type of constraint can be used to control excessive drawdown (equation 3) or mounding (equation 4) of the piezometric surface.

A second type of constraint can be used to place requirements on the difference in heads, between two locations during a stress period;

hi1,t−hi2,t³di,t	(5)

where di,tis the specified bound on head differences and hi1,tand hi2,tare heads at locations i1 and i2 at the end of stress period t. This constraint can be used to force a gradient in the hydraulic flow field, either horizontally or vertically.

### Constraints on Pumping

The primary decision variable in problems solved by MODOFC are the pump rates at specified locations.  These pumping rates may involve either extraction of water from the aquifer or injection (recharge) of water into the aquifer.  In the discussion that follows, the variable qj,t  is used to represent pumping. The index j represents a particular specified location in the MODFLOW domain.  It may be a single cell or a vertical series of cells to represent a well screened in multiple layers.  The index t indicates the stress period during which the pumping rate will remain constant.  Note that MODOFC also allows for decision variables to include groups of stress periods over which pumping remains constant. The variable qj,tmay represent either extraction or injection pumping.  It is implicit that there may be some stress periods during which the well at location j is not an available candidate for pumping.  For these cases, its value is assumed to be zero, or the constraint at the particular location and stress period combination is assumed to be inactive.  When necessary, two additional variables are defined: qj,terepresents pumping at extraction wells, qj,tirepresents pumping at injection wells.  All pumping variables are defined as positive quantities with those conversions needed to account for differences in flux direction accomplished within MODOFC.

#### Pumping Bounds

Constraints are commonly placed upon withdrawal and recharge rates to express bounds on well yield or limitations on the rate at which water can be recharged.  Bounds can be imposed on flux rates as follows;

0≤qj,t≤qju	(6)

where qjuis the maximum extraction or injection rate that can be achieved at location j.  Note that, to solve the location problem, the lower bound must be zero to allow the optimization solver the opportunity to specify that the well has not been selected.

#### Net Pumping Constraint

The total amount of pumping which occurs in a stress period can be limited.  These constraints take the form;

jÎEqj,te−jÎIqj,ti≤Qtu	(7)

jÎEqj,te−jÎIqj,ti³Qtl	(8)

where E is the set of all well locations where extraction is allowed,  I is the set of all well locations where injection is allowed, Qtu is the maximum total pumping in stress period t, and Qtlis the minimum total pumping in stress period t.

#### Recharge Balance Constraints

A defined ratio between extraction and injection can be maintained.  These constraints take the form;

jÎEqj,te³ajÎIqj,ti	(9)

jÎIqj,ti³bjÎEqj,te	(10)

where a and b are recharge balance ratios.  These constraints require that the recharge balance described by a and b is active in any stress period in which pumping occurs.

### Binary Variables

Additional constraints can be imposed on the system by using a series of binary variables, Xj.  These are special variables that are required to take values of either 0 or 1.  In MODOFC, these variables are used to define the status of each well.  If Xjequals 1, candidate well j is considered active (i.e. has been constructed), whereas if Xjequals 0, candidate well j is not active.  The use of binary variables can impose significant additional computational burden (see Section 3.2.6).  It is not recommended that binary variables be used in combination with nonlinear problems (see Section 3.3).

#### Non-zero Lower Bounds on Pumping

With the availability of binary variables, enhanced lower bound constraints can be placed on pumping.  Specifically, it can be required that pumping be either inactive, with zero pumping rate or active but lower bounded by a specified positive value.  Thus, lower bounds on pumping can be expressed as;

qj,t³qjl	(11a)

or

qj,t=0	(11b)

where qjlis the minimum extraction or injection rates that can be achieved at location j.  This either/or type of constraint can be accommodated by using binary variables and replacing (6) with two constraints that take the form:

qj,t≤qjuXj	(12)

qj,t³qjlXj	(13)

When Xj = 1, these constraints act as a simple bounds on pumping with the lower bound able to take on a non-zero value.  When Xj = 0 (well is not active), both bounds become zero forcing the pumping rate to be zero for this well.  These constraints serve both to provide bounds on pumping and to define the value of the binary variable.  If the pumping rate is non-zero, then the binary variable must be one.

#### Constraints on Number of Active Wells

Binary variables can be used to place limits on the number of active wells.  Summing all binary variables to count active wells yields a direct measure of the number of wells needed;

jÎEÈIXj≤Nu	(14a)

and;

jÎEÈIXj³Nl	(14b)

where Nuand Nlare the upper and lower bounds on the number of candidate wells to be used.

#### Constraints on Streamflow Depletion

When the STR package (Prudic, 1989) is included in a simulation, constraints can be imposed on the change in streamflow from a base condition.  The constraint takes the form

Fk,t0−Fk,tq≤Dk,tu	(15a)

Fk,t0−Fk,tq≥Dk,tl	(15b)

Where Fk,tqis simulated streamflow resulting from stresses described by q.  The subscript k indicates a particular segment and reach and t indicates a stress period. Fk,t0is the streamflow at segment/reach k, at the end of stress period t under when none of the managed wells are pumping.  This value is calculated by the simulation model.  The differences on the left side of (15a) and (15b) represent the depletion of streamflow at location k and time t that results from pumping.   The right hand sides of (15a) and (15b) represent the user-specified upper and lower bounds on allowed depletion.

### Objective Functions

MODOFC seeks to satisfy constraints on hydraulic head and pumping rates while  minimizing an objective function defined by the user. Two basic objective function forms are available; linear function of pumped volume and linear function of number of active wells.  These objective function forms can be used to represent, respectively, operating costs or withdrawal volumes and construction costs.

#### Operating Costs or Withdrawal Volumes

The first objective function form is a weighted sum of pumping rates given as;

jÎEÈIt=1Tβjqj,tDtt	(16)

where βj is the cost (e.g. dollars) per unit volume pumped at location j and Dtt is the length of stress period t.  Hence, (16) represents the total cost of pumping.  The use of this cost function form presumes that the cost of operation of the pumping system is linearly proportional to the volume pumped.  The quantity Dttis automatically introduced into (16) by MODOFC using the PERLEN variable in MODFLOW  (see Section 4.3). Since both extraction and injection pumping are defined to be positive, costs associated with these values should be defined with positive values of  βj.  For problems were the objective is to maximize withdrawals or injection, then negativeβjare used.

#### Construction Costs

The second objective form involves the weighted sum of binary variables and is given as:

jÎEÈIkjXj	(17)

where kj is a coefficient. If kj is defined as the installation cost of the candidate well at location  j then (17) represents the cost of installing the wells.  Objective terms (16) and (17) can be added to create a single objective function which represents both construction and operation costs as is demonstrated below.

### Complete Statement of Available Formulation

Combining all of these design elements and repeating over multiple locations and stress periods results in a formulation as follows (equations 3, 4, 5, 7, 8, 9, 10, 13, 14, 15, 16 and 17).

minimizejÎEÈI(t=1Tβjqj,tDtt+kjXj)	(18)

such that

hi,t³hi,tl	iÎL,t=1,K,T	(19)

hi,t≤hi,tu	iÎU,t=1,K,T	(20)

hi1,t−hi2,t³di,t	iÎD,t=1,K,T	(21)

jÎEqj,te−jÎIqj,ti≤Qtu	t=1,K,T	(22)

jÎEqj,te−jÎIqj,ti³Qtl	t=1,K,T	(23)

jÎEqj,te³ajÎIqj,ti	t=1,K,T	(24)

jÎEqj,ti³bjÎIqj,te      	t=1,K,T	(25)

qj,t≤qjuXj	jÎEÈI,t=1,K,T	(26)

qj,t³qjlXj	jÎEÈI,t=1,K,T	(27)

jÎEÈIXj≤Nu		(28a)

jÎEÈIXj³Nl		(28b)

Xjis binary	jÎEÈI	(29)

Fk,t0−Fk,tq≤Dk,tu	k∈K,t=1,…,T	(30a)

Fk,t0−Fk,tq≥Dk,tl	k∈K,t=1,…,T 	(30b)

where U is the set of locations at which upper bounds are placed on the heads, L is the set of locations at which lower bounds are placed on the heads, D is the set of locations at which requirements on the difference in heads are imposed, K is the set of segment/reach location at which streamflow requirements are imposed and T is the total number of stress periods.  Many of the constraints described in the sections above are applied separately in each stress period and over many candidate well locations.  The range of possible index values for each constraint are indicated by the notation to the right of each constraint.  It is understood that the constraint need not be active for all possible combinations of stress periods and candidate wells. If binary variables are not used, then the second term in (18) is dropped, constraints (28)-(29) are eliminated, the binary variables in (26) and (27) are presumed to take the value of 1 and the lower bound on pumping, qj,tl, in constraint (27) must be zero.

### Solving Linear Optimal Flow Control Problems

If the streamflow constraint (30) is not used and the simulated problem is unconfined then the resulting formulation is linear.   The formulation can then be translated into a mixed binary linear program and solved using the simplex algorithm in conjunction with the branch and bound algorithm.  A primary element of this translation is the application of the first order Taylor series expansion to the relationship between head and pumping described by (2). The head at specific time, t, depends on pumping at all locations during stress period tand at all previous stress periods. Statement of the first-order Taylor series yields;

hi,t=hi,t0+t=1tjÎEÈI¶hi,t¶qj,tqj,t+error	(31)

where hi,t0is the head found at location i and stress period t under non-pumping conditions and error is the sum of all truncated high order terms in the Taylor series.  The derivatives in (31) are referred to as response coefficients.

For groundwater problems in which the saturated thickness is fixed (confined aquifers) or the saturated thickness changes relatively little (unconfined aquifers with little drawdown), analysis of equation (1) reveals that the relationship between h and q is linear.  For this case, the error term in (31) is zero and the first order Taylor series approximation is exact.

MODOFC determines the values of ¶hi,t¶qj,t by the method of perturbation.  This method involves a forward difference approximation of the derivative. MODFLOW is used to simulate the pumping of each well, j, by a user-specified increment from a base value.  The response coefficient is computed by measuring the change in head at each location i and stress period t that results from the increment of pumping and calculating the ratio of change in head to change in pumping.  With the values of ¶hi,t¶qj,t and equation (31), the head at any location and stress period can be calculated as a function of pumping without further need of the flow simulator.  Hence, the response coefficients, once computed, provide a complete coupling between simulator and optimizer.

#### Standard Form of Mixed Binary Problem

Substituting (31) into those constraints that depend on head, excluding the streamflow constraints which will be addressed in section 3.3, the formulation (18)-(29) can be written in a standard matrix form as;

minimize ctx+dty	(32)

such that

Axy=b	(33)

xi³0               	 i	(34)

yj is binary        	 j	(35)

where A is a matrix which depends on the response coefficients determined for equation (31) and the constraint statements defined by equations (22-28), b is a vector of right-hand-side constants associated with constraints (19-28), c is a vector of pumping cost coefficients, d is a vector of construction costs, x is the vector of pump rates augmented by the slack and surplus variables which are used to transform the inequalities into equalities, and y is a vector of binary variables.  If  binary variables are not used, the terms involving y disappear and the optimization problem defined by (32-34) is linear and can be solved by the simplex algorithm as described in Appendix B.

#### Solving Mixed Binary Problems

If binary variables are included, then the optimization problem defined by (32-35) is a linear mixed binary program which can be solved by the branch and bound algorithm.  This algorithm solves a series of linear programs within each of which the binary variables are either forced to a binary value or allowed to vary between zero and one (relaxed). Comparisons between the solutions of each of these linear programs leads to a solution without the necessity of checking all possible combinations of binary variable values  (Hillier and Lieberman, 1995; Nemhauser and Wolsey, 1988).

## Nonlinear Optimal Flow Control Problem

In MODOFC Version 2.3, nonlinear response of constraints to pumping can arise when the aquifer is unconfined or when streamflow depletion is constrained.   For unconfined aquifers the saturated thickness changes with pumping. This change in thickness produces an effective change in layer transmissivity which further affects drawdown. This feedback produces a nonlinear relationship between pumping and head in unconfined aquifers.  For problems where streamflow is constrained, the nonlinearity implied by the solution for stream stage, seepage and streamflow in the STR package leads to a nonlinear response of streamflow to pumping.  The presence of these nonlinearities motivates the use of an iterative solution algorithm.

### Solution Algorithm for Nonlinear Problems

The key observation used in solution of the linear flow control problem is that the error term in the Taylor series (31) is zero.  This is no longer true when the aquifer is unconfined.  Similarly, a first order approximation of the streamflow response to pumping will contain some error.   For the case of heads, MODOFC approaches this problem by assuming that the first order approximation is accurate in the vicinity of the current solution point.  The Taylor series can be rewritten as

hi,t'k+1=hi,t'k+t=1t'jÎEÈI¶hi,t'¶qj,tqj,tk+1−qj,tk+error	(36)

where the superscript k represents an iteration level, so that hk is the head obtained when pumping rates qk are applied.  For values of qk+1 that are close to qk , the error in this approximation is small, and the values of hk+1 predicted by this equation are relatively accurate.  It should be noted that, because head is a nonlinear function of pumping, the derivatives (still referred to as response coefficients) in (36) change with the value of the pumping rates.  Response coefficients for streamflow are computed in an analogous way.

#### Sequential Linear Program Algorithm

Considering a nonlinear optimization formulation reduced to a matrix form as is done in (32)-(34), the constraints can be arranged in the form of;

Akxk+1−xk=bk	(37)

where the matrix Ak again depends on the response coefficients and the other constraints.  The subscript, k, indicates the pumping rates that were used to compute the response coefficients.  To implement this solution method in MODOFC, equation (37) is rearranged such that;

Akxk+1=bk+Akxk	(38)

Akxk+1=bk'	(39)

where bk' is the right hand side adjusted by the previous iteration pump rates  (Riefler and Ahlfeld, 1996).  Equation (39) is in the form of a linear program and can be solved using the simplex algorithm.  Because the response coefficients (and the coefficient matrix) depend on the pumping rates, iterations must be used to achieve a solution. At each iteration, an approximate pumping solution is obtained by solving (39).  This approximate pumping solution is then used to recompute the response coefficients and reconstruct Ak. Solving (39) with the newly formulated coefficient matrix produces a new approximate pumping solution.  The algorithm proceeds by repeating this process until pumping rates no longer change from one iteration to the next.   Note that as the iterations converge, the changes in pumping between iterations decrease, and the approximation in (36) becomes increasingly accurate.

#### Algorithm Control: Base Pumping Level and Perturbation Level

At each iteration of the sequential linear program algorithm the response matrix is recomputed using a first order difference approximation.  This approximation requires specification of a base pumping level and a perturbation level.

The default approach to selecting the next base pumping level is to use the most recent optimal solution to the approximate problem.  For the unconfined case, an alternative approach is used which attempts to account for the inaccuracy in the drawdown calculations.  This is described in detail in section 3.3.2.2.

The perturbation level also must be specified and its value can influence the precision of the response coefficient values. Generally, the perturbation computation requires small increments in pumping rate to achieve satisfactory accuracy. However, the perturbation must be large enough to produce several significant digits in the response coefficients.  If the perturbation increment is too small, round-off error in computation of the head may limit the precision of the response coefficient. The number of significant digits in the response matrix is also controlled by the convergence criteria set for the solver in the MODFLOW package (see Section 4.3).

Riefler and Ahlfeld (1996) have done extensive computational work and found that if the perturbation value is too high, then MODFLOW may dewater the aquifer during a perturbation calculation.  Conversely, if the perturbations are too small, the linear approximation obtained may underestimate the drawdown that can be obtained in a single iteration.  In either of these cases, MODOFC may report an infeasible problem even though a solution may exist.  	MODOFC

allows the user to control the size of the perturbation increment.  Problems with infeasible solutions should be examined by considering alternate perturbation increments.  In addition, the size of the perturbation value can affect the speed of convergence.  The impact of perturbation value when streamflow depletion is constrained has been examined by Minihane and Ahlfeld (2002).  MODOFC allows the user to start with a large perturbation amount and reduce it by a scaling factor as the iterations proceed (see Section 4.4.1).

#### Limitations When Solving Nonlinear Problems

MODOFC searches for a local minimum in the pumping domain. For large problems, the solution space may be nonconvex.  Hence, there is no guarantee that the local minimum is the global minimum.  	For complex nonlinear problems MODOFC may be unable to find any solution even when one exists.  If MODOFC cannot find a solution to an unconfined problem, it may still be possible that the problem is feasible.

Solving nonlinear problems with binary variables in the formulation is not recommended.  The presence of the binary variables makes the search for the local minimum more difficult and may lead to suboptimal solutions or spurious infeasible results.

Other nonlinear relationships between pumping and head may arise when using MODFLOW. These include cases where the head drops below river bed elevations, where the head drops below drain elevations or where evapotranspiration characteristics change.  MODOFC is not capable of addressing these sources of nonlinearity.

### Avoiding Dewatering During Unconfined Iterations

Accommodating MODFLOW solutions that contain dewatering is problematic for several reasons. Once dewatering of a cell occurs, the flow regime changes abruptly, thus producing a nonlinear (and possibly discontinuous) relationship between pumping and head.  In addition, MODFLOW may have difficulty rewetting a dewatered cell.  MODOFC is formulated to avoid dewatering of cells.

During the course of the algorithm, it is possible that the approximate pumping solution computed during a given iteration will cause a portion of the aquifer to dewater, when that pumping solution is used to recompute the response coefficients.  Because of the approximate nature of the response coefficients, the approximate pumping solution found at each iteration may not be physically meaningful.  Only the solution obtained at the convergence of the iterations should be considered meaningful.

#### Lower Bound Constraints at Pumping Wells

To prevent the selection of a spurious solution which dewaters the aquifer and aid in the convergence of the algorithm, MODOFC automatically sets lower bound constraints at all extraction wells in unconfined units.  The parameter used for this purpose is BMIN (see section 4.4.1).  If pumping at the candidate wells could cause dewatering elsewhere in the model, the user may manually set lower bound constraints in the sensitive areas. Circumstances may arise in which MODOFC reports that a certain problem is infeasible.  While the problem may in fact be infeasible, it is also possible that the constraints that are preventing feasibility are those imposed to prevent dewatering.  In this latter case, a solution may be available, if dewatering is allowed.

#### Adjustment of Pumping Rates

An additional adjustment to the pumping rates of the approximate solution is often required to avoid dewatering the aquifer.  If an approximate pumping solution, produced by solving (39), brings the head to the lower bound used to prevent dewatering in the confined approximation, it is likely that this pumping scheme will dewater cells when the unconfined aquifer is simulated.  This is because as drawdown reduces the transmissivity (hK) in the unconfined aquifer, the head drop per unit extraction increases.  Thus, the confined approximation applied to the unconfined case tends to overpredict pumping, resulting in excessive drawdown and possible dewatering of the aquifer.  To prevent this, a fraction is used to scale down the approximate pumping solution;

Dqj,tk+1=αj,tkDqj,test	(40)

where;

Dqj,tk+1=qj,tk+1−qj,tk	(41)

Dqj,test=qj,test−qj,tk	(42)

where αj,tk is the scaling fraction for well j,t at the current iteration, qj,tk+1 is the scaled pumping for well j at the current iteration, qj,tkis the pumping used for well j,t at the previous iteration, and qj,test is the optimal pumping computed for well j,t from the solution of (39) at the current iteration.  Combining these terms results in an expression for revising the approximate pumping solution to avoid dewatering;

qj,tk+1=1−αj,tkqj,tk+αj,tkqj,test	(43)

Many approaches can be used to determine , the only necessary requirements being that,  (1) dewatering is prevented, and (2) as the iterations progress the approximate solution converges to the true solution.  MODOFC calculates an  for each well at each iteration using an assumption based on the Theim equation.  It is assumed that the pumping rate required to achieve a given drawdown is proportional to the saturated thickness, b, of the unit.  Thus, the increase in pumping can be scaled back by the estimated change in b;

αj,tk=bj,testbj,tk=hj,test−mjhj,tk−mj	(44)

where mj is the elevation of the bottom of the layer at well j,t.  The head resulting from the approximate pumping solution can be estimated with the response matrix as;

hi,test»hj,tk+aj,tkqest−qk	(45)

where aj,tk is the row associated with the lower bound constraint on well j,t in the response matrix for the current iteration.  This equation is approximate, because it predicts the response of a confined aquifer, not an unconfined aquifer.  Combining equations (44) and (45) results in;

αj,tk=hj,tk−mj+aj,tkqest−qkhj,tk−mj	(46)

Equations (43) and (46) are used in combination to reduce the approximate pumping solution at each iteration.  This serves to prevent spurious dewatering and allows the algorithm to converge to a solution.

# Using MODOFC

This section describes the MODOFC source code and notes on its compilation.  A complete description of input files required for MODOFC is presented.  A description of output files and their interpretation is provided.

## MODOFC Source Code

The MODOFC package consists of a series of files written in FORTRAN 77 each of which contains a number of subroutines. The file ‘modofc23.for’ contains the main program, the branch and bound algorithm subroutine, and the subroutines which construct the optimization problem.  The file ‘ofcio23.for’ contains all input and output subroutines

The MODOFC package contains the entire MODFLOW-96 version 3.0 source code.  File ‘modflw96.for’ contains the main program of MODFLOW-96 modified to be called as a subroutine and with added utility subroutines related to repeated calls of MODFLOW.  File ‘bas5.for’ contains a set of MODFLOW-96 subroutines which has been modified to perform certain input/output functions and pass flow balance information to the file ‘modflw96.for.’  The remainder of the MODFLOW-96 source code is contained in the file ‘modsub.for.’  These subroutines are identical to the source code distributed by the United States Geological Survey with the exception that they have been modified so that all variables are double precision.  All modifications made to the MODFLOW-96 files are in lowercase characters for easy identification.

Once the linear program is formulated, its solution is determined using the simplex algorithm contained in module ‘lpsub.for.’  Additional documentation of ‘lpsub.for’ is provided in Appendix B.

### Dimensioning

All dimensioning is performed in the beginning of file ‘modofc.for’ using parameter statements.  Internal comments describe the meaning of each parameter.  These values should be checked and updated if necessary before compiling the source code and running simulations.  All of the dimensioning parameters have internal error checking to insure that dimension size is sufficiently large.  The main storage array in MODOFC is named ‘xm1’.  This storage space is used by the linear programming solver and is also used by MODFLOW during its execution.  It should be dimensioned large enough to accommodate both needs.

### Compilation of MODOFC

To run MODOFC all the source code (including the provided MODFLOW-96 source code) must be compiled into one executable file.  All the necessary source code is identified by the suffix “.for”.

## MODOFC Input Features

### Capture Zones

A feature of MODOFC is the capability for automatic generation of head difference constraints by specification of the outline of a capture zone.   The user specifies the coordinate locations of a series of connected line segments and the head gradients that will be forced across those lines.  The program converts these into a series of head difference constraints which encompasses the entire capture zone.  Multiple disconnected capture zones can be specified.  Sample Problem I demonstrates this feature.

### Multiple Well Screens

Sometimes it is necessary to consider wells which are screened over multiple layers in the groundwater model.  This can represent wells that are screened in different geologic formations or wells which are screened across a single formation that has been modeled as multiple layers.  In MODOFC, single wells can be defined which extract or inject water at defined ratios to the different model units.  Only a single composite pumping amount, the sum over all of the screens, is considered in the optimization program.  The user must define the layers over which the well is screened and the ratios which rations the total pumping amongst the layers.  It is suggested that the pumping over each screen should be proportional to the screen length times the hydraulic conductivity of the unit.  Sample Problem I demonstrates this feature.

### Repeated Optimization

MODOFC also contains a feature to conduct rapid repeated optimization runs with linear optimization problems.  The slowest step in running MODOFC is typically the computation of the response matrix by repeated calls to MODFLOW.  To save computer time for successive optimization runs,  MODOFC can store the elements of the response matrix to an unformatted file called ‘response.’  Successive runs can then be conducted by reading in the response matrix rather than computing it.

This procedure will only return correct results if the response matrix remains valid and the basic formulation has remained unchanged. First, there can be no change in the MODFLOW input files.  The simulation model must be identical from one set of runs to the next. Second the size of the formulation must remain the same.  There can be no change in the location and available stress period for the candidate wells, head bound constraints, head difference constraints and capture zones and streamflow constraints.

Changes can be made in the well bounds, well costs, head bound values, head difference values, streamflow depletion values, total pumping constraints, recharge balance constraints, bounds on the number of wells installed, or gradients across capture zone line segments.  In addition, wells, head bounds, head difference constraints, streamflow constraints, total pumping constraints, or capture zones can be made unavailable by setting the appropriate I/O flag for the specific feature as described in the input file directions.  While these problem features can be made unavailable, the input line associated with them must remain present in the main input file.

This feature can not be used for unconfined aquifers problems. In this case, the response matrix is recomputed repeatedly during the course of a single optimization run.

## MODFLOW Input Files

It is assumed that the user of MODOFC has already constructed, calibrated, and verified a site-specific model for groundwater flow.  The MODFLOW input files required for the flow model, need to be provided, unmodified, for the MODOFC optimization.  The flow model may contain any of the features of MODFLOW-96.    It should be observed that all MODFLOW features, except for streams modeled with the STR package, such as rivers, drains, etc. are assumed to respond linearly to pumping over the entire range of pump rates expected to be encountered during solution of the problem.  A well package file must be provided and the MXWELL variable must be set to at least the number of background wells plus the total number of candidate wells.  For multiple stress period simulations, MODOFC requires that starting heads be saved (ISTART = 1).

The quantity Dtt in (16) is automatically introduced by MODOFC using the PERLEN variable in MODFLOW.  When multiple stress periods are used, the product of pumping and stress period length in (16) gives the total volume of fluid pumped in location j and stress period  t.  For single stress period problems Dtt all pumping rates are multiplied by the same stress period length making the specific value of Dttirrelevant to the solution.  Nevertheless, MODOFC continues to use the value of PERLEN for the specified stress period.  While this value is not relevant to MODFLOW calculations, it is retained in the MODOFC objective function, and the reported pumping costs include this time factor.

Solving nonlinear problems requires small perturbations to determine accurate response matrices (see Section 3.3.1.2).  At low perturbations, head needs to be calculated with great precision by MODFLOW in order to insure adequate precision in the response matrix.  For sample problems using a perturbation of 10 ft3/day, a convergence criteria on the order of 1 x 10-8 was necessary to obtain four significant digits for head difference constraints in the response matrix.  This value is problem dependent and may not be effective for all problems.  The average number of significant digits in the response matrix is printed to an output file (see Section 4.5).  Testing has shown that for most problems, only the preconditioned conjugate gradient solver can achieve this level of precision (Riefler and Ahlfeld, 1996).  In addition, for MODOFC to accurately estimate the precision of the response matrix, MODFLOW convergence criteria must be based on change in head (HCLOSE).

If there is a chance that a layer in the problem will be dewatering during simulations, it is necessary that MODFLOW be capable of simulating flow with a thin saturated thickness.  It is recommended that the default harmonic mean averaging scheme be changed to one of the averaging schemes more accurate for unconfined aquifers (Goode and Appel, 1992).  Testing with simple problems revealed that using the arithmetic mean averaging scheme (LAYCON = 11) convergence was achieved in homogenous units with saturated thicknesses as small as 2 feet.  It may also be helpful to enable the rewetting feature of MODFLOW, although this option is not recommended since simulations may have difficulty recovering from a cell dewatering (McDonald et al., 1992).

## Input File

The optimization input is contained in a single input file, ‘opt.in,’ which fully describes the optimization formulation.  All items in ‘opt.in’ are read in free (or list-directed) format, so no adherence to column location is necessary.  The units of values in ‘opt.in’ should be consistent with the units used in the user’s data set for MODFLOW.  At execution the ‘opt.in’ file must be in the same directory as the MODOFC executable.  Example ‘opt.in’ files are provided in Appendix A.  The ‘opt.in’ file is divided into eight sections.  The first section contains the main information defining the optimization problem, the second section defines the candidate wells, the third section defines the recharge balance constraints, the forth section defines the total pumping constraints, the fifth section defines the head bound constraints, the sixth section defines the head difference constraints, the seventh section defines the capture zone constraints, and the eighth section defines streamflow depletion constraints.

### Main Section

The first 20 lines of the file describe the optimization formulation and directions for its solution.  Input values should appear in the following order:

Line Number	Entries

1		PROBNM

2		NUMWELS

3		NPUMP

4		NCNODES

5		NGNODES

6                 NSNODES

7		NCZ

8		X0	Y0	RDEGR

9		ICOM

10		ITMAX

11		PERTI	PERTF	PERTS

12		BMIN

13		OCC

14		ITPR

15		IECHO

16		NPRT(1,1)		NPRT(1,2)	NPRT(1,3)

17		NPRT(2,1)		NPRT(2,2)	NPRT(2,3)

18		NPRT(3,1)		NPRT(3,2)	NPRT(3,3)

19		NPRT(4,1)		NPRT(4,2)	NPRT(4,3)

20		NAMES

PROBNM (character)

This line contains the user-defined problem name.  This name will be written to several of the output files for identification purposes.  This line can have a maximum length of 72 characters.

NUMWELS (integer)

This value indicates the number of candidate wells from which MODOFC can select an optimal pumping solution.  This number must be consistent with the number of lines provided in the well section.  Note that this number should not be changed, if wells are made unavailable, and that the variable MXWELLS in the MODFLOW well input file must be set greater than or equal to the number of managed wells plus the number of unmanaged wells.

NPUMP (integer)

This value indicates the number of total pumping constraints (equations 14 and 15) that need to be satisfied by the optimization program.  This number must equal the number of input lines given in the total pumping constraint section (excluding the DUMMY lines).  Note that this number should not be changed, if head bound constraints are made unavailable.

NCNODES (integer)

This value indicates the number of head bound constraints (equations 3 and 4) that need to be satisfied by the optimization program.  This number must equal the number of input lines given in the head bound constraints section (excluding the DUMMY lines).  Note that this number should not be changed, if head bound constraints are made unavailable.

NGNODES (integer)

This value indicates the number of head difference constraints (equation 5) that need to be satisfied by the optimization program.  This number must equal the number of input lines given in the head difference constraints section (excluding the DUMMY lines).  Note that this number should not be changed, if head difference constraints are made unavailable.

NSNODES (integer)

This value indicates the number of streamflow depletion constraints (equation 15) that need to be satisfied by the optimization program.  This number must equal the number of input lines given in the streamflow constraints section (excluding the DUMMY lines).  Note that this number should not be changed, if streamflow constraints are made unavailable.

NCZ (integer)

This value indicates the number of capture zones (not line segments) that MODOFC will use to create a series of head difference constraints.  For MODOFC, a capture zone is defined as a series of connected line segments across which a gradient will be forced.  This number must be consistent with the number of capture zones defined in the capture zone section (see section 4.4.7).

X0, Y0 (real numbers)

These values describe the location of the origin of the coordinate system for capture zone specification.  MODOFC assumes that the MODFLOW coordinate system has its origin at the lower left corner of the grid (at the first column and the last row).  Decreasing the row index means moving in a positive y-direction and increasing the column index means moving in a positive x-direction.  Setting X0 and Y0 equal to zero keeps the origin at the lower left corner of the lower left cell in the grid.  These numbers are ignored if NCZ = 0.

RDEGR (real number)

This value indicates the angle of rotation (in degrees) the capture zone grid will have relative to the MODFLOW grid.  A positive value indicates a rotation in the clockwise direction.  Setting RDEGR equal to zero maintains the same grid orientation. This number is ignored if NCZ = 0.

ICOM (integer)

This value can only be set equal to 0, 1, or 2. When equal to 0, MODOFC reads the response matrix from the file ‘response’ produced in a previous run.  When equal to 1, MODOFC calculates the response matrix and stores it in the file ‘response.’ When equal to 2, MODOFC calculates the response matrix but does not save it. This feature can greatly reduce the time required to solve MODOFC. When solving unconfined problems, this value is automatically set to 2.  This feature can only be used when the limited changes are made to the optimization formulation.  See Section 4.2.3 for detailed discussion of use of this feature.

ITMAX (integer)

This value is the maximum number of iterations allowed for the linear program solver.  This prevents the solver from iterating indefinitely if it does not converge to a solution.  If ‘lpsub.for’ is being used and this number is reached, the program will be terminated, and the file ‘solution’ will indicate that the maximum number of iterations has been exceeded.  A typical value for ITMAX is ten times the number of constraints.

PERTI, PERTF, PERTS (real numbers)

These values control the perturbation value used to determine the response matrix.  The values input here are scaling factors and are multiplied by the upper bound on pumping rate to determine the actual perturbation value used at any iteration.  PERTI is the scale factor used for the first iteration, PERTF is the scale factor used for the last iteration, and PERTS is a parameter defined so that on the second iteration the perturbation equals PERTIPERTS, on the third iteration the perturbation equals PERTIPERTS2, etc.  When the calculated scale factor perturbation is below PERTF, PERTF is used.  For linear simulations, only one iteration occurs so PERTF and PERTS are not used.  Under linear conditions the choice of PERTI is arbitrary and should not affect results. A recommended value for the perturbation parameter under linear conditions is 500% of expected solution magnitude.   This can be achieved by setting PERTI and the pumping upper bound to appropriate values.   For nonlinear simulations, PERTI should be selected so that the perturbation value is of the same order of magnitude as the solution.  PERTF should be selected so that the perturbation value is large enough to produce several significant digits in the response matrix but small enough to secure an accurate approximation for the nonlinear response.  The number of significant digits in the response matrix is also controlled by the convergence criteria set for the solver in the MODFLOW package (see Section 4.3).  Typical values for PERTI and PERTF for a nonlinear problem are set so that the resulting perturbation value is 100% of expected solution magnitude and 0.5% of expected solution magnitude, respectively.  See discussion in Section 3.3.1.2 for more information.

BMIN (real number)

This value indicates the minimum saturated thickness allowed in unconfined units.  It is necessary to avoid dewatering portions of the unconfined units.  This value is used to automatically set lower head bound constraints at all extraction wells in unconfined units.  These constraints are named ‘b’ plus the corresponding well name.   If there are no unconfined units in the system, this value is ignored.  A typical value for BMIN is 15% of the saturated thickness (see section 3.3.2.1 for discussion).

OCC (real number)

This value indicates the convergence criteria for solving unconfined problems.  Iteration stops when the maximum difference in pumping rates between the previous iteration and the current iteration is below OCC.  If there are no unconfined units in the system, this value is ignored.  This value is typically set at the precision desired for the pumping solution.

ITPR (integer)

This value determines whether output describing the details of the unconfined iteration algorithm will be directed to file ‘iterate’.  A 1 indicates this output will be created and a 0 indicates it will not.  For nonlinear problems with many wells, this file can be very large.  See Section 4.5 for a description of the output produced.  If there are no unconfined units in the system, this value is ignored.

IECHO (integer)

IECHO determines whether the input read by MODOFC from file ‘opt.in’ will be echoed to file ‘setup’. A 1 indicates that input will be echoed, a 0 that input will not be echoed.

NPRT (integer array)

NPRT determines what MODFLOW output is printed to the named listing output file.  The first column represents echo printing of the MODFLOW input, the second column represents convergence information, and the third column represents head and flow balance information.  The first row represents the initial call to MODFLOW, the second row represents the call to MODFLOW with the previous pumping solution, the third row represents the calls to MODFLOW to determine the current response matrix, and the fourth row represents the final call to MODFLOW with the optimal solution.  Setting an array element greater than zero indicates that information corresponding to the column will be printed to the output file at the step corresponding to the row.  For rows 2 and 3, setting the entry to greater than 1 indicates that the information will only be printed at iterations evenly divisible by the entry.  Note that echo printing of input can only be used for rows 1 and 4.  A typical NPRT for a confined simulation is:

1	0	1

0	0	0

0	1	0

0	1	1

A typical NPRT for an unconfined simulation is:

1	0	1

0	2	0

0	2	0

0	0	1

NAMES (character)

This input defines the MODFLOW names file.  The variable is read in free format and has a maximum length of 20 characters.  The file is normally located in the same directory as the MODOFC executable.   The format for this file is described in Harbaugh and McDonald (1996).

### Candidate Wells Section

The next set of lines describe the candidate wells.  Input values should appear in the following order:

Line Number	Entries

1		DUMMY

2		INCST

3		INPR

4		NWMIN		NWMAX

5		DUMMY

6	WLN	I/O	NR	NC	NL	NUML	I/E	PMN	PMX	PCST	ICST	SP

7		RAT		NML

8		RAT		NML

...		...		...

...	...	...	...	...	...	...	...	...	...	...

...	WLN	I/O	NR	NC	NL	NUML	I/E	PMN	PMX	PCST	ICST	SP

DUMMY (character)

This line is not read by MODOFC but is simply a place holder to separate one section from another and to provide space for comment.

INCST (integer)

This flag determines whether binary variables are to be included in the optimization problem.  The use of binary variables complicates the solution procedure, but enables incorporation of installation costs and constraints on the number of wells selected (NWMIN, NWMAX) and on minimum pumping rates (PMN).  If INCST equals 0, binary variables are not included.  If INCST equals 1, binary variables are included.  Sample Problem II demonstrates the inclusion of binary variables.

INPR (integer)

This value determines whether output describing the details of the brand and bound algorithm for solving mixed binary problems will be directed to file ‘iterate’.  A 1 indicates this output will be created and a 0 indicates it will not. For problems with many wells, this file can be very large.  This value is not used if INCST equals zero.  See section 4.5 for a description of the output produced.

NWMIN, NWMAX (integers)

These values set the minimum and maximum number of wells the solution can use, Nl in (14b) and Nu in (14a).  These values are not used if INCST equals zero.  Setting NWMIN equal to zero and NWMAX equal to NUMWELS eliminates restrictions on the number of wells.

WLN (character)

A unique name designation of ten characters or less must be provided for each well.  No spaces are allowed in the name.  The end of the name is designated by a blank space.

I/O (character)

This is a flag which determines whether the following candidate well will be considered by the optimization algorithm.  A value of “t” indicates the well is available, and a value of “f” indicates the well is unavailable.  If the well is unavailable then no pumping will occur at the well. Uppercase or lowercase input are acceptable.  For linear optimization, this can be used to remove a well from the candidate set without having to recalculate the response matrix (ICOM = 0).

NR, NC, NL (integers)

These values designate the location of the candidate well using MODFLOW grid coordinates.  Note that in the MODFLOW coordinate system, the upper left node in the top layer is location (1,1,1).

NUML (integer)

This indicates the number of layers over which the well is screened.  If NUML equals 1, the well is screened in one layer and the next input line should start a new well.  If NUML > 1, the previous value of NL is ignored and the next NUML lines are read in to find the ratio (RAT) and layer (NML) for each screen segment.  Sample Problem I demonstrates the use of wells screened over multiple units.

RAT (real number)

This weights the percentage of total pumping which is transmitted through the screen in the indicated layer.  The flow in each screened interval is suggested to be proportional to the length of screen times the hydraulic conductivity of the unit.  If all of the values of RAT for a well do not sum to unity, MODOFC computes fractions for each layer by dividing each value by the sum.  This should only be provided if NUML > 1.

NML (integer)

This indicates the layer in which each well screen exists.  This should only be provided if NUML > 1.

I/E (character)

This is a flag which determines the direction of flux at the well.  A value of “i” or “r” (for recharge) indicates that only injection is allowed, and a value of “e” indicates that only extraction is allowed.  To allow extraction or injection at a candidate well, two lines of input are required.  Uppercase or lowercase input are acceptable.

PMN (real number)

This value indicates the minimum pumping rate allowable for the candidate well, qjlin (13).  Note that this constraint does not force a well to be pumping, rather the well can be set to zero or between qjland qju. If INCST equals zero this value is not used, and the minimum pumping rate is assumed equal to zero.

PMX (real number)

This value indicates the maximum pumping rate allowed at the candidate well, qju in (6) and (12).  Note that upper bounds that are different with each stress period can be defined by defining a “new” well for each new set of bounds.  Each “new” well consists of a new input line with the same grid coordinate location.  However, a separate binary variable will be defined for each “new” well. It is not recommended that this procedure be used when binary variables are employed.

PCST (real number)

This value indicates the cost per unit volume pumped at the candidate well, j in (16).  If negative values are used, the objective becomes a maximization of the weighted sum of pumping.

ICST (real number)

This value indicates the cost for installation at the candidate well, j in (17).  This value is not used if INCST equals zero.

SP (character)

This character string defines the stress periods over which the well is available.  The entry can be up to 120 characters long and must be enclosed by single quotes.  The string must not contain any blank spaces.  Wells can be forced to maintain the same pump rate for multiple stress periods or be allowed to have different pump rates for multiple stress periods.  Stress period groups for which wells have different pump rates are separated by colons (:).  Stress periods for which wells must pump at the same rate are separated by commas (,) or hyphens (-).  For example:

‘1,3’	well pumps at the same rate for stress periods 1 and 3

‘1-4’	well pumps at the same rate for stress periods 1 - 4

‘1:2:3:4’	well pumps at different rates for stress periods 1 - 4

‘1,3:2,4’	well pumps at one rate for stress periods 1 and 3 and another rate for stress periods 2 and 4

For steady-state problems, an entry of ‘1’ is required for each well.  Sample Problem III demonstrates the use of multiple stress periods.

### Recharge Balance Constraints Section

These three lines define the recharge balance constraints defined in equations (9) and (10).  These constraints produce both upper and lower bounds on the ratio of extraction and injection, and are imposed separately for each stress period in which both injection and extraction wells are available.  The same upper and lower bound ratios are imposed in each applicable stress period.  Note that these constraints will take the form of a specified ratio between extraction and injection, if the coefficients are defined so that b=1a.   The constraints will be consistent with each other as long as ab≤1.  Input values should appear in the following order:

Line Number		Entries

1		DUMMY

2		A

3		B

DUMMY (character)

This line is not read by MODOFC but is simply a place holder to separate one section from another and to provide space for comment.

A (real number)

This value indicates the fraction of total injection which total extraction must exceed, a in equation (9). If A equals zero, the constraint is not imposed.

B (real number)

This value indicates the fraction of total extraction which total injection must exceed, b in equation (10). If B equals zero, the constraint is not imposed.

### Total Pumping Constraints Section

These lines define the total pumping constraints defined in equations (7) and (8).  Input values should appear in the following order:

Line Number		Entries

1		DUMMY

2		DUMMY

3		PNAME	I/O	PSP	MINP	MAXP

...		...		...	...	...	...

NPUMP+2		PNAME	I/O	PSP	MINP	MAXP

DUMMY (character)

This line is not read by MODOFC but is simply a place holder to separate one section from another and to provide space for comment.

PNAME (character)

A unique name designation of ten  characters or less must be provided for each constraint.  No spaces are allowed in the name.  The end of the name is designated by a blank space.

I/O (character)

This is a flag which determines whether the following total pumping constraint will be considered by the optimization algorithm.  A value of “t” indicates the constraint is imposed, and a value of “f” indicates the constraint is not imposed.  Uppercase or lowercase input are acceptable.

PSP (integer)

This indicates the stress period during which the constraint will be imposed.

MINP (real number)

This indicates the minimum total pumping that will be required for the stress period, Qtl in equation (8).  Extraction is defined as positive pumping and injection as negative pumping.  Thus, MINP can be viewed as the minimum total extraction allowed or -MINP as the maximum total injection allowed.  MAXP must be greater than or equal to MINP.

MAXP (real number)

This indicates the maximum total pumping that will be required for the stress period, Qtu  in equation (7).  Extraction is defined as positive pumping and injection as negative pumping.  Thus, MAXP can be viewed as the maximum total extraction allowed or -MAXP as the minimum total injection allowed.  MAXP must be greater than or equal to MINP.

### Head Bound Constraints Section

The next set of lines describe the head bound constraints (3) and  (4).  Input values should appear in the following order:

Line Number	Entries

1		DUMMY

2		DUMMY

3		CTNAME	I/O	NROW	NCOL	NLAY	U/L	BND	SP

...		...	...	...	...	...	...	...	...

NCNODES+2	CTNAME	I/O	NROW	NCOL	NLAY	U/L	BND	SP

DUMMY (character)

This line is not read by MODOFC but is simply a place holder to separate one section from another and to provide space for comment.

CTNAME (character)

A unique name designation of ten  characters or less must be provided for each constraint.  No spaces are allowed in the name.  The end of the name is designated by a blank space.

I/O (character)

This is a flag which determines whether the following head bound constraint will be considered by the optimization algorithm.  A value of “t” indicates the constraint is imposed, and a value of “f” indicates the constraint is not imposed.  Uppercase or lowercase input are acceptable.

NROW, NCOL, NLAY (integers)

These values designate the location of the head bound constraint using MODFLOW grid coordinates.  Note that in the MODFLOW coordinate system, the upper left node in the top layer is location (1,1,1).

U/L (character)

This is a flag which determines whether the constraint is an upper bound or lower bound.  A value of “<” indicates an upper bound described by equation (4), and a value of “>” indicates a lower bound described by equation (3).

BND (real number)

This value designates the bounding value, hi,tl or hi,tu in (3 and 4).

SP (character)

This character string defines the stress periods over which the constraint is imposed.  The entry can be up to 120 characters long and must be enclosed by single quotes. The string must not contain any blank spaces.   Multiple stress periods for which the constraint is imposed are separated by commas (,) or a hyphen (-).  For example:

‘1,3’	constraint is imposed for stress periods 1 and 3

‘1-4’	constraint is imposed for stress periods 1 - 4

For steady state problems, an entry of ‘1’ is required for each constraint.  Sample Problem III demonstrates the use of multiple stress periods.

### Head Difference Constraint Section

The next set of lines describe the head difference constraints (5).  Input values should appear in the following order:

Line Number	Entries

1	DUMMY

2	DUMMY

3	CTNAME	I/O	NR2	NC2	NL2	NR1	NC1	NL1	HD	SP

...	...		...	...	...	...	...	...	...	...	...

NGNODES+2	CTNAME	I/O	NR2	NC2	NL2	NR1	NC1	NL1	HD	SP

DUMMY (character)

This line is not read by MODOFC but is simply a place holder to separate one section from another and to provide space for comment.

CTNAME (character)

A unique name designation of ten  characters or less must be provided for each constraint.  No spaces are allowed in the name.  The end of the name is designated by a blank space.

I/O (character)

This is a flag which determines whether the following head difference constraint will be considered by the optimization algorithm.  A value of “t” indicates the constraint is imposed, and a value of “f” indicates the constraint is not imposed.  Uppercase or lowercase input are acceptable.

NR2, NC2, NL2, NR1, NC1, NL1 (integers)

These values designate the location of the second point, i2 in (5), in the head difference constraint (NR2, NC2, NL2) and the first point, i1 in (5), in the head difference constraint (NR1, NC1, NL1) using MODFLOW grid coordinates.  Note that in the MODFLOW coordinate system, the upper left node in the top layer is location (1,1,1).  This constraint requires that the head at the point listed first (i2) be lower than the head at the point listed second (i1) by a specified amount.

HD (real number)

This value specifies the required head difference, di,t in (5).

SP (character)

This character string defines the stress periods over which the constraint is imposed.  The entry can be up to 120 characters long and must be enclosed by single quotes.  The string must not contain any blank spaces.  Multiple stress periods for which the constraint is imposed are separated by commas (,) or a hyphen (-).  For example:

‘1,3’	constraint is imposed for stress periods 1 and 3

‘1-4’	constraint is imposed for stress periods 1 - 4

For steady state problems, an entry of ‘1’ is required for each constraint.  Sample Problem III demonstrates the use of multiple stress periods.

### Capture Zone Constraints Section

The next set of lines describes the capture zone constraints.  MODOFC translates the series of lines segments described below into head difference constraints of the form of equation (5). These head difference constraints are automatically named in MODOFC with the prefix ‘C’ followed by a five digit code.  The first two digits give the number of the line segment.  The last three digits give the cumulative number of generated head difference constraints.  A maximum of 99 line segments and 999 generated head difference constraints should be used. Sample problem I demonstrates the capture zone feature.  Input values should appear in the following order:

Line Number	Entries

1	DUMMY

2	NCZS	LAYER	SP

3	CAPX	CAPY	GRAD

...	...	...	...

NCZS+2	CAPX	CAPY

Note:  lines 2 to NCZS+2 must repeat for the number of capture zones specified (NCZ)

DUMMY (character)

This line is not read by MODOFC but is simply a place holder to separate one section from another and to provide space for comment.

NCZS (integer)

This value indicates the number of line segments (not the number of points) in the capture zone.  The number of capture zone points will be NCZS + 1.

LAYER (integer)

This value indicates the layer in which the capture zone is placed.  Capture zones cannot cross layers, although separate capture zones can be created in different layers.

SP (character)

This character string defines the stress periods over which the entire capture zone is imposed.  The entry can be up to 120 characters long and must be enclosed by single quotes.  The string must not contain any blank spaces.  Multiple stress periods for which the capture zone is imposed are separated by commas (,) or a hyphen (-).  For example:

‘1,3’	capture zone is imposed for stress periods 1 and 3

‘1-4’	capture zone is imposed for stress periods 1 - 4

For steady state problems, an entry of ‘1’ is required for each capture zone.

CAPX, CAPY (real numbers)

These values designate the location of the points in the capture zone.  Line segments are drawn from the previous point, to the current point, to the following point.  At least two points are required for a capture zone.  These numbers identify the points using the capture zone coordinate system defined by X0, Y0, and RDEGR.  The distance is in MODFLOW distance units (not grid points).

GRAD (real number)

This value specifies the gradient required across the line segment between this point and the next point.  A GRAD value for the last point is ignored.  Note that the gradient is the head difference divided by the distance between nodes.  The inside of the capture zone is defined as right of the line when moving from point to point.  If GRAD is positive, heads right of the line will be required to be less than heads left of the line.

### Streamflow Depletion Constraints Section

The next set of lines describe the streamflow depletion constraints (15).  Input values should appear in the following order:

Line Number	Entries

1		DUMMY

2		DUMMY

3		CTNAME	I/O	SEG   REACH	U/L	BND	SP

...		...	...	...	...	...	...	...	...

NSNODES+2	CTNAME	I/O	SEG   REACH	U/L	BND	SP

DUMMY (character)

This line is not read by MODOFC but is simply a place holder to separate one section from another and to provide space for comment.

CTNAME (character)

A unique name designation of ten  characters or less must be provided for each constraint.  No spaces are allowed in the name.  The end of the name is designated by a blank space.

I/O (character)

This is a flag which determines whether the following streamflow constraint will be considered by the optimization algorithm.  A value of “t” indicates the constraint is imposed, and a value of “f” indicates the constraint is not imposed.  Uppercase or lowercase input are acceptable.

SEG, REACH (integers)

These values designate the location of the head bound constraint using MODFLOW grid coordinates.  Note that in the MODFLOW coordinate system, the upper left node in the top layer is location (1,1,1).

U/L (character)

This is a flag which determines whether the constraint is an upper bound or lower bound.  A value of “<” indicates an upper bound described by equation (15a), and a value of “>” indicates a lower bound described by equation (15b).

BND (real number)

This value designates the bounding value of streamflow depletion, Dk,tl or Dk,tu in (15a and 15b).  The units of streamflow depletion are the same as those used in the STR package input.

SP (character)

This character string defines the stress periods over which the constraint is imposed.  The entry can be up to 120 characters long and must be enclosed by single quotes. The string must not contain any blank spaces.   Multiple stress periods for which the constraint is imposed are separated by commas (,) or a hyphen (-).  For example:

‘1,3’	constraint is imposed for stress periods 1 and 3

‘1-4’	constraint is imposed for stress periods 1 - 4

For steady state problems, an entry of ‘1’ is required for each constraint.  Sample Problem III demonstrates the use of multiple stress periods.

## Output Files

MODOFC produces up to five output files.  Under normal usage, MODOFC will produce three main output files: ‘setup’, ‘iterate’, and ‘solution’.  Additional files are the MODFLOW listing file and the response coefficient file.

The output file, ‘setup’, contains information related to the initiation of the problem solution which will vary depending on the nature of the input files.  This information includes input echo, details of how the capture zone constraints are translated into head difference constraints (see Section 4.2.1), dimensioned storage needed and that which is available, the status of the constraints when there is no non-background pumping, the magnitude of the perturbation and the precision available in the constraint coefficients (see Section 3.3.1.2).  When the problem is unconfined and iterations are required this information is provided in the ‘iterate’ file.

The output file ‘iterate’ contains detailed information on the progress of the iterations required to solve the unconfined and integer problems.  The algorithm used to solve the unconfined problem is described in Section 3.3. When the problem is unconfined ‘iterate’ will contain, at each iteration, the bounds and computed heads at each extraction well.  Also reported are the pumping rate correction terms described in equation (46).  The column labeled “Optimal q” is the optimal solution determined by the linear program at each iteration of the solution algorithm.  See sample problem I for output produced under this condition. Output to the file ‘iterate’ can be eliminated by setting  ITPR = 0 (see section 4.4.1).

When the formulation contains binary variables the file ‘iterate’ will contain each solution computed during execution of the branch and bound algorithm. See Sections 3.2.3 and 3.2.6.2 for details on when the problem will contain binary variables and how the branch and bound algorithm works. The solutions are designated as relaxed, current best solution, feasible, or infeasible.  For each solution, the well, its optimal value and the status of its binary variable are reported. See sample problem II for output produced under this condition. This output can be eliminated by setting INPR = 0 (see Section 4.4.2).

The main results from MODOFC are printed to the file ‘solution’.  It lists the solution status, the objective value, the active pump locations and rates, the cost associated with the pumping rates selected, the binding constraints and their shadow prices, and a detailed range analysis. The section on binding constraints shows the type, name and location of those constraints that are binding.  In addition, the shadow price (or dual variable) is shown for each constraint when available and applicable. Shadow prices represent the local sensitivity of the objective function value to changes in the right-hand side value of a particular constraint.  The shadow price of nonbinding constraints is zero.

Range analysis provides information about the local behavior of the optimal solution and its sensitivity to changes in parameter values.  Range analysis finds the range within which a cost coefficient or a right-hand side value can lie without changing the optimal basis. In a groundwater flow control problem, range analysis determines the range over which a drawdown or gradient constraint can change without changing the set of active pumping wells. Range analysis is performed under the assumption that the problem is linear and that all variables are continuous between their upper and lower bounds. The constraint range analysis lists the slack (distance of the constraint from its binding value), the shadow price and the original right hand side value. Constraint range analysis also determines the range of RHS values for which the basis remains unchanged.  Although the set of active decision variables does not change, the optimal values of these variables will change. The cost coefficients range analysis list the reduced cost and original cost coefficients. Cost coefficient range analysis also indicates the amount a cost coefficient can change without an effect on the optimal solution.

The output produced by MODFLOW-96 is directed to the MODFLOW-96 listing file defined in the MODFLOW names file.  The contents of this file is determined by the NPRT array described in Section 4.4.1.

Using the ICOM input variable (see Section 4.4.1), MODOFC can be directed to save the response matrix. This is accomplished by writing the unformatted file ‘response’ to disk.   Substantial computational time can be saved by reuse of the response matrix. This file should be retained if additional MODOFC runs are to be made using the same formulation.  In this situation, the file ‘response’ becomes an input file to MODOFC.  This file can be discarded if the response matrix is no longer needed.

## Execution

To execute MODOFC type the executable filename in a directory that has all the necessary input files.  These files are the MODFLOW input files described in Section 4.3, the optimization input file named ‘opt.in,’ described in Sections 4.4 and the file ‘response,’ if the response matrix is being read.  MODOFC will then create in the active directory the files described in Section 4.5.

# Sample Problems

Three samples problems have been included in this documentation to demonstrate the features of MODOFC.  Sample Problem I demonstrates a groundwater containment scenario in which head bounds, head difference constraints, and a capture zone.  In this problem, wells screened over multiple intervals and unconfined units are used.  Sample Problem II demonstrates a construction site dewatering project in which head bounds are used with construction costs in a confined aquifer.  Binary variables and the branch and bound algorithm are used for this problem.  Sample Problem III demonstrates a water supply problem in which drawdown and minimum total extraction is constrained.  In this problem, the simulations use multiple stress periods.

## Sample Problem I:  CONTAIN

This sample problem represents a hazardous waste site in which contaminated groundwater must be contained.  The sample problem has two layers, an upper unconfined unit above a lower confined unit with lengths of 600 feet and widths of 400 feet.  Both layers have no flow boundary conditions along the top and bottom boundaries, fixed heads of 50 feet along the left boundary, and fixed heads of 45 feet along the right boundary.  The layers are horizontally homogenous and isotropic with a vertical conductance between the two of 0.05 1/day.  The upper unconfined unit has a hydraulic conductivity of 5 ft/day, a bottom elevation of 30 feet, and a uniform recharge of 0.002 ft/day.  The lower confined unit has a transmissivity of 800 ft2/day.  Each layer is divided into 600 square block centered nodes each 20 ft by 20 ft.  A production well is located in the lower unit at grid point (13,20,2) pumping at 10,000 ft3/day.  The grids for these layers are shown in Figures 1 and 2 in Appendix A.

It is assumed that an area in the upper layer of the model domain has been contaminated.  From a design standpoint, at steady state, all contaminated water must be contained within a capture zone, while the production well continues to operate.  Mounding above the ground surface cannot occur at injection wells (assumed at 65 feet), and the upper unit cannot be dewatered.  Due to surface features at the site, there are only 6 possible well locations as shown in Figure 1 and well 5 is screened in both units.  These design criteria can be converted into linear programming constraints, using head difference constraints and head constraints.  First, a capture zone of three line segments has been defined around the contaminated area with a required gradient of 0.01 ft/ft.  MODOFC will convert this into 23 head difference constraints.  In addition, head difference constraints requiring a head difference of 0.01 ft have been defined in the source area to ensure upward vertical gradients and prevent contaminated water from leaking into the lower unit.  A fixed head constraint has been used at the candidate injection well requiring that head remain below 65 feet.  Finally, a minimum hydraulic thickness of 2 feet was allowed in the unconfined aquifer.  It was assumed that extraction costs 5 times more than injection due to treatment costs.

This sample problem demonstrates several features of MODOFC.  First, all possible head constraints are employed:  head constraints, head difference constraints, and capture zones.  Note the coordinates of the capture zone, which use the MODFLOW system of the origin at the upper left location of the grid, down is the positive Y-direction, and to the right is the positive X-direction.  Because lower heads are required on the right side of the capture zone line, the first node is defined near Well 1.  MODFLOW input file distances are used, for this case each cell is 20 ft by 20 ft.  This problem also has an unconfined layer in it, so the solution will be determined through iteration, and care must be given to defining the first perturbation (PERTI), last perturbation (PERTF), perturbation scaling (PERTS), minimum aquifer thickness (BMIN), iteration convergence criteria (OCC), and whether iterations should be printed (ITPR).  Also, repeated optimizations are not possible (ICOM = 1) and inclusion of binary variables is not recommended (INCST = 0) with unconfined problems.  Because INCST = 0, values for installation costs (ICST), minimum pumping rates (PMN), and minimum (NWMIN) and maximum (NWMAX) number of wells to install are ignored.  Wells 2 and 4 are screened over multiple units in this problem.  Note the syntax for Wells 2 and 4, i.e. NL = 0 and extra rows are provided to set the pumping ratios for each screened layer.

A solution was reached in 14 iterations.  The input and output files are provided in Appendix A.  Note that the preconditioned conjugate gradient solver was used with a head change convergence criteria of 1 x 10-8, and that the arithmetic mean was specified for the averaging scheme in the unconfined unit (LAYCON(1) = 11).  It is recommended that the user run this sample problem to ensure that the same results are achieved with their compiler.  Execution of the sample problem requires MODFLOW input files defined with the filenames ‘contain.bas,’ ‘contain.rch,’ ‘contain.bcf,’ ‘contain.nam,’ ‘contain.wel,’ and ‘contain.pcg’ and the MODOFC input file ‘opt.in.’.  Execution produces progress reports to the screen and files ‘contain.lst’, ‘setup’, ‘iterate’, ‘solution,’ and ‘response’.  The distribution files ‘setup.con’, ‘iterate.con’, ‘solution.con’ and ‘lst.con,’ and should be identical (baring round-off error due to compiler differences) to the corresponding files produced in a test run.

## Sample Problem II:  DEWATER

This sample problem represents a dewatering problem for a construction site.  The sample problem has one confined layer with a length of 3,000 feet and a width of 2,000 feet.  The layer has boundary conditions of no flow along the top and bottom boundaries, fixed head of 80 feet along the left boundary, and fixed head of 60 feet along the right boundary. The layer is homogenous and isotropic with a transmissivity of 50 ft2/day.  The aquifer is divided into 600 square block centered nodes each 100 ft by 100 ft.  The grid is shown in Figure 3 in Appendix A.

In this problem, the groundwater needs to be lowered to a depth of 50 feet so that footings can be installed in the region shown on Figure 3.  This design criteria can be imposed by defining 10 upper bound head constraints as shown in Figure 3.  Seven candidate well locations are selected with minimum pumping rates of 100 ft3/day, maximum pumping rates of 10,000 ft3/day, pumping costs for the construction period of 10 $ day/ft3, and construction costs of $2000 per well.  To improve reliability, at least three wells must be installed.

This sample problem primarily demonstrates the use of binary variables.  To include the cost for well installation, binary variables must be used, thus INCST = 1.  This enables several other constraints, so meaningful values must be provided for installation costs (ICST), minimum pumping rates (PMN), and minimum (NWMIN) and maximum (NWMAX) number of wells to install.  Also, if the user wants to record branch and bound algorithm solution steps, INPR must be set to 1.

The optimal solution was determined after the branch and bound algorithm solved eight different linear programs.  Input and output files are provided in Appendix A.  Note that range analysis and some shadow price information is not available when binary variables are used.  It is recommended that the user run this sample problem to ensure that the same results are achieved with their compiler.  Execution of the sample problem requires MODFLOW input files defined with the filenames 'dewater.bas', 'dewater.bcf', 'dewater.nam', 'dewater.wel' and 'dewater.pcg' and the MODOFC input file 'opt.in'.  Execution produces progress reports to the screen and files ‘contain.lst’, ‘setup’, ‘iterate’, ‘solution,’ and ‘response’.  The distribution files ‘setup.dew’, ‘iterate.dew’, ‘solution.dew’ and ‘lst.dew,’ and should be identical (baring round-off error due to compiler differences) to the corresponding files produced in a test run.

## Sample Problem III:  SUPPLY

This sample problem represents a water supply problem in which limited drawdown is allowed near a river.  The sample problem has one confined layer, 6,000 feet by 5,000 feet, with a river running through the center of it.  The layer has boundary conditions of no flow along the left and right boundaries, fixed heads along the top boundary which vary from 100 feet to 86 feet, and fixed heads along the bottom boundary which vary from 80 feet to 66 feet.  The layer is homogenous and isotropic with a transmissivity of 5000 ft2/day and a storativity of 0.05.  The aquifer is divided into 750 square block centered nodes each 200 ft by 200 ft.   The stream is 20 feet wide with a bed conductance of 20,000 ft3/day and slope of 0.0025.  The stream is modeled with three segments.  Two upstream segments join to form a single downstream segment.  Recharge also varies throughout the year:  0.0005 ft/day in winter, 0.002 ft/day in spring, 0 ft/day in summer, and 0.001 ft/day in fall.  The grid and stream are shown in Figure 3 in Appendix A.

In this problem, the objective is to maximum groundwater extraction over a three-year period (12 seasons) while maintaining a certain level of water in the stream.  This design criteria can be imposed by defining upper head constraints in the groundwater at locations under the stream, as shown in Figure 4.  In addition, there are limits on the maximum (80,000 ft3/day) and minimum (30,000 ft3/day in the first year and 25,000 ft3/day in the second and third year) total rates of water extracted in a three-month period.  These values define the total pumping constraints.  Four possible well locations are shown in Figure 4 with maximum pumping rates at each well of 50,000 ft3/day and pumping costs of -0.001 $ /ft3/day (i.e., each 1000 cubic foot of water pumped is worth $1).  The wells and constraints are only active for various stress periods as shown in the opt.in listing in Appendix A.

This sample problem primarily demonstrates optimization of groundwater problems with multiple stress periods.  Because the recharge and streamflows vary with the season, a transient groundwater model is required to properly depict the site.  It may be advantageous to have wells pump at different rates for each season.  This is exhibited in Well 2 which is required to have one pump rate for winter (stress periods 1, 5, and 9), another for spring (stress periods 2, 6, and 10), another for summer (stress periods 3, 7, and 11) and another for fall (stress periods 4, 8, and 12).  Well 1 on the other hand is required to have the same pump rate for the entire 3 years (stress periods 1-12).  Well 3 will only be pumping in year 2 (stress periods 5-8) and Well 4 can only pump in spring (stress periods 2, 6, and 10) and fall (stress periods 4, 8, and 12).  Similarly, constraints are only active for certain stress periods:  b-01 and b-03 for the entire 3 years (stress periods 1-12) and b-03 and b-04 for summer (stress periods 3, 7, and 11) and fall (stress periods 4, 8, and 12).

An optimal solution was found, and input and output files are provided in Appendix A.  It is recommended that the user run this sample problem to ensure that the same results are achieved with their compiler.  Execution of the sample problem requires MODFLOW input files defined with the filenames 'supply.bas', 'supply.bcf', 'supply.out', 'supply.wel', ‘supply.rch’, ‘supply.nam’, ‘supply.riv’ and 'supply.pcg' and the MODOFC input file 'opt.in.' Execution produces progress reports to the screen and files ‘contain.lst’, ‘setup’, ‘iterate’, ‘solution,’ and ‘response’.  The distribution files ‘setup.sup’, ‘iterate.sup’, ‘solution.sup’ and ‘lst.sup,’ and should be identical (baring round-off error due to compiler differences) to the corresponding files produced in a test run.

# References

Ahlfeld, D. P., and A. E. Mulligan, 2000, "Optimal Management of Flow in Groundwater Systems", Academic Press, San Diego, CA.

Ahlfeld, D. P., R. H. Page, and G. F. Pinder.  1995.  Optimal ground-water remediation methods applied to a Superfund site:  From formulation to implementation.  Groundwater.  Vol. 33.  No. 1.  p. 58-70.

Goode, D. J. and C. A. Appel.  1992.  Finite-difference interblock transmissivity for unconfined aquifers and for aquifers having smoothly varying transmissivity.  U.S. Geological Survey Water Resources Investigations Report 92-4124.

Harbaugh, A. W. and M. G. McDonald.  1996.  User’s documentation for MODFLOW96, an update to the U.S. Geological Survey modular finite-difference ground-water flow model.  U.S. Geological Survey.  Open-File Report 96-485.

Hillier, F. S. and G. J. Lieberman.  1995.  Introduction to Operations Research.  McGraw-Hill, Inc.  New York, NY.  p. 494-541.

McDonald, M. G. and A. W. Harbaugh.  1988.  A modular three-dimensional finite-difference ground-water flow model.  U.S. Geological Survey Techniques of Water-Resources Investigations Book 6, Chapter A1, 586.

M.R. Minihane and D.P. Ahlfeld, “Application of Management Models to Water Supply with Stream/Aquifer Interactions”, Proc. 2002 Conference on Water Resources Planning and Management, American Society of Civil Engineers, Roanoke, VA, May 19-22, 2002.

Nemhauser, G. L., and L. A. Wolsey, 1988, Integer and Combinatorial Optimization, Wiley, New York.

McDonald, M. G., A. W. Harbaugh, B. R. Orr, and D. J. Ackerman.  1992.  A method of converting no-flow cells to variable-head cells for the U.S. Geological Survey modular finite-difference ground-water flow model.  U.S. Geological Survey.  Open-File Report 91-536.

Prudic, D.E., 1989, “Documentation of a computer program to simulate stream-aquifer relations using a modular, finite-difference, ground-water flow model”, U.S. Geological Survey Water-Resources Investigations Report 88-729, 113 p.

Riefler, R. G. and D. P. Ahlfeld.  1996.  The impact of numerical precision on the solution of confined and unconfined optimal hydraulic control problems.  Haz. Waste Haz. Mater.  Vol. 13.  No. 2.  pp. 167-176.

Wagner, B. J., 1995, Recent advances in simulation-optimization groundwater management modeling, Reviews of Geophysics, Supplement, 33, pp. 1021--1028.