MODOFC 3.0

August 2006

Optimization coupled with MODFLOW-96

MODOFC formed the basis for a new, expanded groundwater management program.  This new program is called GWM (Ground Water Management) and is a MODFLOW-2000 process.  A MODFLOW 2005 version of GWM is also available.  GWM code and all documentation can be obtained at http://water.usgs.gov/nrp/gwsoftware/mf2k-gwm/MF2K-GWM.html.

MODOFC 2.3 and earlier versions are no longer supported.  Instead, MODOFC 3.0 has been created.  It utilizes the packages of GWM along with the input structure and output capabilities of GWM.  MODOFC 3.0 retains the programming structure of calling MODFLOW-96 as a subroutine.

Users of MODOFC 3.0 should primarily refer to the documentation for GWM to understand MODOFC 3.0.  The input and output for MODOFC 3.0 is described in the GWM documentation with one exception, as described below.  Input instructions in MODOFC 2.3 documentation should be ignored.  Capture zone and automated placement of drawdown constraints at wells are no longer supported.  GWM summation constraints generalize some of the capabilities found in MODOFC 2.3.  The capabilities and theory underlying MODOFC 3.0 are primarily described in the GWM documentation.  The user may wish to refer to the MODOFC 2.3 documentation for information related to coupling with MODFLOW-96.

# Input to MODOFC 3.0

MODOFC 3.0 relies almost entirely on the input structure used in GWM.  The exception to this is identification of several files.  MODOFC 3.0 reads the names of three files from the ‘opt.in’ file.  The ‘opt.in’ file must have the name ‘opt.in’.  The file names are read in the order shown in the table below.  The table also summarizes the difference between the location where MODOFC and GWM obtain these file names.   Unit numbers for these files are internally assigned by MODOFC.  The unit number 93-97 are reserved for defining input and output files.  See the sample problems for examples of the opt.in file.

| MODOFC File | Corresponds to GWM File | GWM reads it from: | MODOFC reads it from: |
| --- | --- | --- | --- |
| MODOFC Output File | MODFLOW Global File | MODFLOW Name File | Opt.in |
| MODFLOW Name File | MODFLOW Name File | Screen | Opt.in |
| GWM Input File | GWM File | MODFLOW Name File | Opt.in |

Sample problems:

MODOFC 3.0 is distributed with several sample problems.  These are described in both the MODOFC 2.3 documentation and the GWM documentation.  The input files for the sample problems differ only in the input files for the Groundwater Flow Process.   The input files for the Groundwater Management Process follow the requirements defined in the GWM documentation.  MODOFC 3.0 input files are compatible with MODFLOW-96.  The sample problems available are:

Dewater: a head constrained, linear problem

Dewatermb: same as Dewater, except binary variables are used.

Supply2: a streamflow constrained, transient problem.

The input and output files for each sample problem can be found in the SAMPLES directory distributed with MODOFC 3.0.  The Contain problem, distributed with prior versions of MODOFC is no longer supported.

Source Code Distribution:

This distribution package contains only two program units: MODOFC3_0.for and MODSUB.for.  The first contains the MODOFC main program, several utility routines, the MODFLOW-96 main program (modified to be called as a subroutine) and several other MODFLOW-96 subroutines.  The second program unit contains the remaining subroutines of MODFLOW-96.

To compile MODOFC3.0 following GWM program units are required.

GWM1BAS1.for

GWM1BAS1SUBS.for

GWM1DCC1.for

GWM1DCV1.for

GWM1HDC1.for

GWM1OBJ1.for

GWM1RMS1.for

GWM1RMS1MOD.for

GWM1SMC1.for

GWM1STC1.for

These can be downloaded at the GWM web location cited above.  Download GWM and look in the src directory for the latest versions of these program units.

MODOFC 3.0 has been compiled using Compaq Visual Fortran v. 6.5.  The executable distributed with MODOFC 3.0 has been compiled with the GWM Version 1.1 set of program units.

Questions about MODOFC 3.0 should be directed to David Ahlfeld:  ahlfeld@ecs.umass.edu